# [ASEN 5044] # Statistical Estimation of Dynamical Systems # Progress Report 2 Fall 2020 Group Members: Joshua Malburg Junior Sundar

# Question #4

## Implementing Linearized Kalman Filter (LKF)

To implement the LKF for this Cooperative Localization problem we define an LKF function which performs the prediction [Equation ( 1 )] and correction [Equation ( 2 )] step.

The function takes in the ground truth values (state dynamics and measurements), the nominal state and measurement trajectories without process or measurement noise, the process noise covariance (QKF) for tuning, and initial values for δx+0 and P+0. The function outputs the state estimates (xestimate=xnominal+δx+), measurement estimates yestimatek=Hk+1δxk+1- and estimation errors (ex=xtruth-xestimate and ey=ttruth-yestimate).

|  | δxk+1-=Fkδxk+ Pk+1-=FkPk+FKT+ΩkQkΩkT | ( 1 ) |
| --- | --- | --- |

|  | Sk+1-=Hk+1Pk+1-Hk+1T+Rk+1 Kk+1=Pk+1-Hk+1TSk+1--1 δxk+1+=δxk+1-+Kk+1δyk+1-Hk+1δxk+1- Pk+1+=In-Kk+1Hk+1Pk+1- | ( 2 ) |
| --- | --- | --- |

This function is included in a Monte Carlo run which feeds the ground truth models, nominal state and measurement trajectories, and the DT state space matrices linearized around the nominal trajectories (Fk,Hk,Ωk) according to the methods from Question #2 and Question #3.

## Truth Model Testing (TMT) for LKF

For the TMT we do 50 Monte Carlo (N=50) runs. This is an appropriate number of Monte Carlo runs as we will have enough data sets for performing an unbiased NEES and NIS test.

The truth model is a simulated run of the nonlinear model with the process and measurement noise generated using the covariances uploaded on Canvas. We seed xtruth0 for the ground truth values for every Monte Carlo run from dx0~0,P+0 and then xtruth0=xnominal+dx0 and then for every subsequent values the process noise is obtained from covariance matrix ‘Qtrue’.

Additionally, for the multiple runs in the TMT we feed the LKF the following initial values:

|  | δx+0=xnominal0-xtruth0 P+0=diag1,1,0.025,1,1,0.025 | ( 3 ) |
| --- | --- | --- |

These initial conditions were selected for the following reasons: we are given the nominal and truth state trajectories, and hence the initial perturbation can be derived from those values; furthermore, we have a sufficient degree of certainty of our initial state perturbation such that the state perturbation covariance is finite (P+0≠∞).

In addition to this, in the LKF, the covariance matrix for the measurement noise (R) is set to be the one uploaded on canvas, i.e. Rtrue. As that information is generally known to us from the sensors being used.

## Tuning

There are two aspects that we need to keep in mind while tuning the Kalman filter. First, we need to make sure that the Kalman filter ‘works properly’, i.e. the estimate error averages at 0, and the 2σ bounds are converging. Second, we also look at the NEES and NIS chi-square tests and make sure they are within the confidence intervals.

We calculate the confidence intervals on MATLAB with significance level α=0.01. We chose this significance level to provide a less stringent condition for proving whether the LKF is doing its job (i.e. having a low enough false-alarm probability). This was because we found that the LKF is not good enough for estimating this nonlinear system. The bounds are calculated using the MATLAB function ‘chi2inv’. As a result, for N=50 Monte Carlo runs, n=6 states, p=5 measured values, the chi-square confidence bounds are:

|  | 4.8133≤ϵx,k≤7.3369 3.9232≤ϵy,k≤6.2269 | ( 4 ) |
| --- | --- | --- |

While tuning the LKF we vary our predicted process noise covariance matrix (QKF) until the two conditions in the beginning of this subsection are satisfied. We first start with QKF=0.1I6×6 and run the truth model tests and check if the KF works and the NEES and NIS tests are satisfied. The result showed that the neither of the conditions were met, hence we moved to QKF=I6×6 then to QKF=10I6×6 and tweaked the main diagonal parameters in this way until the conditions were sufficiently satisfied.

In the end, the process noise covariance matrix for the LKF that best satisfied the conditions was found to be:

|  | QKF=diag(1,1,3,1,1,3) | ( 5 ) |
| --- | --- | --- |

Before we move forward, there are some points that need to be highlighted regarding the result. First, the NEES test COULD NOT be satisfied for any selection of QKF regardless of the magnitude of the covariance matrix or selection of initial conditions. The KF was debugged thoroughly, and we could not find any method to alleviate this problem. Second, the NIS test could be satisfied, however periodically at every ~25 seconds the chi-square values exceed the bounds. Although, we managed to bring most of the points for ϵy,k within the bounds in Equation ( 5 ). Finally, it was also found that regardless of the tuning methods employed, the state estimate errors corresponding to North position and East position of the UGV (i.e. eηg and eξg) are unstable and their 2σ bounds do not converge.

The best results obtained after tuning the LKF are shown in the following plots. Figure 1 is a plot showing the NEES and NIS chi-square tests for the Monte Carlo runs with the LKF. Figure 2 is a plot of the state estimate errors from a sample Monte Carlo run with LKF under the same conditions and Figure 3 shows the actual state estimates and the state ground truths for the very same sample run.

First looking at Figure 1, we can see that the NIS test is sufficiently satisfied except for every ~25 seconds where the points are outside the bounds. Upon further investigation, this was found due to the R covariance matrix used for the LKF. In the matrix, the elements directly corresponding to the aircraft’s North position and East position values are greater than the other elements that are a combination of the UGV’s and UAV’s together. For instance, elements R(4,4)=R5,5=36. And the UAV has a control input of ωa=π25rad⋅s-1 which corresponds to a period of 25 seconds. Furthermore, the NEES test is not satisfied and the problem could not be alleviated regardless of the tuning method. This issue was found to be because the LKF could not accurately ascertain the state evolution of the UGV (ηg, ξg and θg).

Also note, that the state estimate errors for ηa and ξa are sufficiently low, except of the time points where the ξa state peaks, and the state estimation covariance (P+) also converges for the UAV’s states. The estimate errors for θa and θg also tend to increase at points where the angles are wrapped to within the -π,π bounds.

Referring to Figure 2 and Figure 3, we can see that ηg and ξg estimates are barely close in periodic intervals. The LKF fails in estimating these two states. Furthermore, another observation that is that the covariance for the state estimates (i.e. Pk+) for  ηg and ξg are oscillating and not converging. Selecting a QKF that alleviates this problem affected the other states adversely, and hence it was left this way. The oscillation also has a period of ~25 seconds and the 2σ bounds for ηg is out of phase with ξg.

## Comment on LKF for Estimating Nonlinear System

The tuning procedure was arduous and complicated. However, after multiple attempts, we can conclude that the LKF is not a good enough filter to estimate the nonlinear system for cooperative localization.

We derived this conclusion because we were not able to tune the LKF to satisfy the two conditions: the LKF ‘works properly’ and satisfies the NEES and NIS chi-square tests.

This section went into the methodology used to implement the LKF and the process used to tune it. We also went into the issues faced when tuning and the compromises and assumptions made.

The LKF failed the NEES test and barely passed the NIS test, indicating that the state estimates obtained from this filter are not close enough to the ground truths. This was even after multiple attempts in tuning the filter.

Referring to the UAV’s states, it seems that the LKF starts to mis-estimate its states when the aircraft is moving near the boundaries of its domain in and ξa. However, at all other points, the LKF provides a decent estimate for the UAV’s state. However, the LKF completely fails to successfully estimate the states for UGV. This is credited to the fact that we can directly observe the UAV’s states, while the UGV’s states are only derived indirectly through trigonometric equations.

The LKF fails to estimate the states ηg and ξg for an extended periods and this in and of itself disqualifies the LKF.

Finally, the 2σ bounds, or covariance, for ηg and ξg fail to converge over time, which also disqualifies the LKF. We tried to fix this problem by modifying the initial conditions and QKF elements, however in the cases where we got the bounds to converge, the bounds for ηa and ξa exhibit the same oscillating property, and the NEES and NIS tests were worse off. Hence we selected the conditions in Equation ( 3 )( 4 )( 5 ) as a compromise.

Finally, we also tested the filter by checking whether angle wrapping between operations effected or improved the results (i.e. wrapping the nominal and ground truths before feeding into LKF). The results were slightly worse off, because the filter would fail to estimate the states at the point where the angle wrapped back to within the -π,π intervals. Hence angles are not wrapped until after the estimate is obtained through the filter.

The MATLAB codes for the LKF implementation, the Monte Carlo runs, and the NEES and NIS tests are attached in Appendix D.

Figure 1 – NEES and NIS chi-square test for N=50 LKF Monte Carlo runs with Equations ( 3 )( 4 )( 5 ) applied

Figure 2 – State estimate errors and 2σ bounds of sample LKF Monte Carlo run with Equations ( 3 )( 4 )( 5 ) applied

Figure 3 – State estimates and state ground truth of sample LKF Monte Carlo run with Equations ( 3 )( 4 )( 5 ) applied

# Question #5

## Extended Kalman Filter (EKF)

The EKF starts off at time-step zero (0) with an estimate of the total state and covariance.  At each time-step k, during the prediction step the EKF uses the non-linear equations to calculate the estimated state which in turn is used to approximate the predicted covariance matrix.

xk+1-=fxk+,uk,wk=0

Pk+1-=FkPk+FkT+ΩkQkΩkT

where			Fkxk+,uk,wk=0≈I+∆T∙A(xk+,uk(tk),wk(tk)=0)

Ωk≈∆T∙Γ(t)(t=tk)

During the correction step the EKF calculates the innovation vector (yk+1-yk+1-) using the measurement estimate derived from the non-linear measurement equation.

yk+1-=hxk+1-,vk+1=0

eyk+1=yk+1-yk+1-

The filter then generates a new linearized DT System (H) matrix, linearized about the new state estimate, to calculate the Kalman gain, update the total state estimate and update the covariance matrix.

Hk+1=δhδxxk+1-

Kk+1=Pk+1-Hk+1THk+1Pk+1-Hk+1T+Rk+1-1

xk+1+=xk+1-+Kk+1eyk+1

MATLAB code for implementation and tuning of the EKF can be found in Appendix E.

## Tuning Approach

Tuning the filter involved generating randomized ground truth data from many Monte Carlo simulations and calculating the mean NEES and NIS values at each time step.  The EKF’s process noise matrix Q was then tweaked until the NEES and NIS consistency tests were within the desired accuracy bounds (alpha = ±5%).  The initial state was also randomized to demonstrate the filter wasn’t biased to a specific nominal trajectory.

## Typical Simulation Plots

The section shows plots from a typical simulation run. Figure 4captures an example of the randomly generated UGV and UAV state data.

Figure 4 - Simulation Run - Ground Truth States

Figure 5 shows an example of the randomly generated noisy measurement data.

Figure 5 - Simulation Run - Simulated Measurement Data

Figure 6 – Estimated State Errors

Figure 7 - Estimated Measurement Errors

## Chi-Square Test Results

This section shows the final NEES and NIS test results.

Figure 8 - Final NEES results

Figure 9 - Final NIS Results

Appendix D – Codes accompanying Question 4

function [x_est,y_est,dx,P,S,e_x,e_y] = LKF(dx_init,P_init,x_nom,y_nom,x,y,Fk,Hk,Ok,Q,R) n = size(Fk(:,:,1),2); p = size(Hk(:,:,1),1); steps = length(x_nom(1,:))-1; dx = zeros(n,steps+1); P = zeros(n,n,steps+1); S = zeros(p,p,steps+1); K = zeros(n,p,steps+1); dx(:,1) = dx_init; dy = y-y_nom; P(:,:,1) = P_init; I = eye(n); e_y = zeros(p,steps+1); e_y(:,1) = y(:,1) - Hk(:,:,1)*(x_nom(:,1) + dx(:,1)); y_est = zeros(5,steps+1); y_est(:,1) = Hk(:,:,1)*(x_nom(:,1) + dx(:,1)); S(:,:,1) = Hk(:,:,1)*P(:,:,1)*Hk(:,:,1)' + R; K(:,:,1) = P(:,:,1)*Hk(:,:,1)'*inv(S(:,:,1)); for i=1:steps % Prediction Step dx(:,i+1) = Fk(:,:,i)*dx(:,i); P(:,:,i+1) = Fk(:,:,i)*P(:,:,i)*Fk(:,:,i)' + Ok(:,:,i)*Q*Ok(:,:,i)'; % Correction Step S(:,:,i+1) = Hk(:,:,i+1)*P(:,:,i+1)*Hk(:,:,i+1)' + R; K(:,:,i+1) = P(:,:,i+1)*Hk(:,:,i+1)'*inv(S(:,:,i+1)); y_est(:,i+1) = Hk(:,:,i+1)*(x_nom(:,i+1) + dx(:,i+1)); e_y(:,i+1) = y(:,i+1) - y_est(:,i+1); dx(:,i+1) = dx(:,i+1) + K(:,:,i+1)*(dy(:,i+1) - Hk(:,:,i+1)*dx(:,i+1)); P(:,:,i+1) = (I - K(:,:,i+1)*Hk(:,:,i+1))*P(:,:,i+1); end x_est = x_nom + dx; e_x = x - x_est; end

function [x_truth,y_truth,x_est,y_est,dx,P,S,e_x,e_y] = LKF_MonteCarlo(Q,R,steps) load("cooplocalization_finalproj_KFdata.mat"); x_nom = [10 0 pi/2 -60 0 -pi/2]'; u_nom = [2 -pi/18 12 pi/25]'; x_pert = [0 1 0 0 0 0.1]'; Dt = 0.1; n = size(x_nom,1); P0 = eye(n).*0; t = 0:Dt:steps*Dt; % Generate truth model outputs for nominal trajectories [x_truth, y_truth] = GenerateTruth(x_nom, u_nom, P0, Qtrue, Rtrue, Dt, steps); % Generate nominal trajectories [~,x_NL] = ode45(@(t,x) NL_DynModel(t,x,u_nom,zeros(6,1)),t,x_nom); x_NL = x_NL'; y_NL = zeros(5,length(t)); for i=1:length(t) y_NL(:,i) = NL_MeasModel(x_NL(:,i),zeros(5,1)); end % Generate DT matrices along nominal trajectory x_nominal = x_NL; y_nominal = y_NL; Fk = zeros(6,6,length(t)); Hk = zeros(5,6,length(t)); Ok = zeros(6,6,length(t)); for i=1:length(t) [A_t, B_t, C_t] = Linearize(x_nominal(:,i),u_nom); [Fk(:,:,i), ~ , Hk(:,:,i)] = Discretize(A_t,B_t,C_t, Dt); Ok(:,:,i) = eye(6); end % Run LKF on Data dx_init = x_pert; P_init = eye(6); [x_est,y_est,dx,P,S,e_x,e_y] = LKF(dx_init,P_init,x_nominal,y_nominal,x_truth,y_truth,Fk,Hk,Ok,Q,R); end

clc clear load('cooplocalization_finalproj_KFdata.mat') seed = 100; rng(seed); Dt = 0.1; steps = 1000; n = 6; p = 5; t = 0:Dt:steps*Dt; N = 50; % No. of Monte Carlo runs NEES_all = zeros(N,steps+1); NIS_all = zeros(N,steps+1); % Tuning parameters % Q(1,1) = 0.5; % Q(2,2) = 0.5; % Q(3,3) = 5; % Q(4,4) = 5; % Q(5,5) = 5; % Q(6,6) = 5; Q(1,1) = 1; Q(2,2) = 1; Q(3,3) = 3; Q(4,4) = 1; Q(5,5) = 1; Q(6,6) = 3; % Q = 100*eye(6); R = Rtrue; for i=1:N disp(i) NEES = zeros(1,steps+1); NIS = zeros(1,steps+1); [x_truth,y_truth,x_est,y_est,dx,P,S,e_x,e_y] = LKF_MonteCarlo(Q,R,steps); for k=1:steps+1 NEES(:,k) = e_x(:,k)'*inv(P(:,:,k))*e_x(:,k); NIS(:,k) = e_y(:,k)'*inv(S(:,:,k))*e_y(:,k); end NEES_all(i,:) = NEES; NIS_all(i,:) = NIS; end NEES_bar = zeros(1,steps+1); NIS_bar = zeros(1,steps+1); for i=1:steps+1 NEES_bar(1,i) = mean(NEES_all(:,i)); NIS_bar(1,i) = mean(NIS_all(:,i)); end alpha = 0.01; rx1 = (chi2inv(alpha/2,N*n))./N; rx2 = (chi2inv(1-alpha/2,N*n))./N; ry1 = (chi2inv(alpha/2,N*p))./N; ry2 = (chi2inv(1-alpha/2,N*p))./N;

Appendix E – Codes accompanying Question 5

% Question 5 - Tune EKF close all, clearvars, clc load("cooplocalization_finalproj_KFdata.mat"); x0 = [10 0 pi/2 -60 0 -pi/2]'; u0 = [2 -pi/18 12 pi/25]'; Dt = 0.1; n = size(x0,1); steps = 1000; seed = 100; rng(seed); Q = diag([.0015, .0015, 0.01, 0.001, 0.005, 0.01]); P0 = diag([1 1 0.025 1 1 0.025]); runs = 100; EX = zeros(n, steps+1, runs); p = 5; EY = zeros(p, steps+1, runs); PS = zeros(n, n, steps+1, runs); SS = zeros(p, p, steps+1, runs); fig1 = figure(1); fig2 = figure(2); fig3 = figure(3); fig4 = figure(4); enablePlotDuring = true; for run = 1:runs disp(['run #', num2str(run)]); % generate truth for run [x, y] = GenerateTruth(x0, u0, P0, Qtrue, Rtrue, Dt, steps, true); t = (0:(length(x)-1))*Dt; % assume we can get exact measurement noise from % specifications of sensors R = Rtrue; % Run filter for all time-steps of run #k [x_est, y_est, P, S] = EKF(x0, P0, u0, y, Q, R, Dt); % wrap angle diff too!! ex = x - x_est; ex(3,:) = angdiff(x_est(3,:),x(3,:)); ex(6,:) = angdiff(x_est(6,:),x(6,:)); ey = y - y_est; ey(1,:) = angdiff(y_est(1,:),y(1,:)); ey(3,:) = angdiff(y_est(3,:),y(3,:)); % Plot error during monte carlo runs if enablePlotDuring == true PlotStates(fig1,t,ex, ['State Errors, Run ',num2str(run)], P); PlotMeasurements(fig2,t,y,'Ground Truth Measurements'); PlotStates(fig3,t,x,'Ground Truth States'); PlotMeasurements(fig4,t,ey,['Ground Truth Measurement Errors, Run ',num2str(run)]); end % save run data from NEES/NIS tests EX(:, :, run) = ex; EY(:, :, run) = ey; PS(:, :, :, run) = P; SS(:, :, :, run) = S; end %% Calculate NEES and NIS statistics [NEES_bar, NIS_bar] = CalcStats(EX, EY, PS, SS); %-------------------------------------------------------------------------- % Plots for (a) PlotStates(fig1,t,x - x_est, ['State Errors, Run ',num2str(run)], P); PlotMeasurements(fig2,t,y,'Ground Truth Measurements'); PlotStates(fig3,t,x,'Ground Truth States'); PlotMeasurements(fig4,t,y - y_est,['Ground Truth Measurement Errors, Run ',num2str(run)]); %-------------------------------------------------------------------------- % Plots for (b) fig5 = figure(5); alpha = 0.05; PlotNees(fig5, NEES_bar, runs, n, alpha); %-------------------------------------------------------------------------- % Plots for (c) fig6 = figure(6); PlotNis(fig6, NIS_bar, runs, p, alpha);

function [x_est, y_est, P, S] = EKF(x0, P0, u, y, Q, R, Dt) % set simulation tolerances for ode45 opt = odeset('RelTol',1e-6,'AbsTol',1e-6); n = size(x0, 1); % number of states p = size(R, 1); % number of measurements steps = size(y,2); % number of time steps x_est = zeros(n, steps); % state estimate vector y_est = zeros(p, steps); % measurement estimate vector P = zeros(n, n, steps); % coveriance S = zeros(p, p, steps); % start with initial estimate of total state % and covariance x_p = x0; P_p = P0; for i=1:steps %---------------------------------------------------------- % Prediction Step % use full NL equations to estimate state at next time step % using state at previous time step; since Wk is AWGN, % its expected value is zero, set input to zero wk = zeros(1,n); [~, x_m] = ode45(@NL_DynModel, [0.0 Dt], x_p', opt, u', wk); x_m = x_m(end,:)'; x_m(3) = wrapToPi(x_m(3)); x_m(6) = wrapToPi(x_m(6)); % to calculate covariance, linearize "online" % about current state estimate [A_t,B_t,C_t] = Linearize(x_m, u); [F, ~, H] = Discretize(A_t, B_t ,C_t, Dt); P_m = F*P_p*F' + Q; x_m(3) = wrapToPi(x_m(3)); x_m(6) = wrapToPi(x_m(6)); % uese estimated state from NL ODEs; since Wk is AWGN, % its expected value is zero, set to zero vk = zeros(p,1); y_est(:,i) = NL_MeasModel(x_m, vk); y_est(1,i) = wrapToPi(y_est(1,i)); y_est(3,i) = wrapToPi(y_est(3,i)); % calculate innovation vector e_y = y(:,i) - y_est(:,i); e_y(1) = wrapToPi(e_y(1)); e_y(3) = wrapToPi(e_y(3)); %---------------------------------------------------------- % Correction Step % calculate gain using linearized measurement % matrix H and covariance from Prediction Step S_p = H*P_m*H' + R; K = P_m*H'/S_p; % calculate posterior state estimate and covariance x_p = x_m + K*e_y; P_p = (eye(n) - K*H)*P_m; x_p(3) = wrapToPi(x_p(3)); x_p(6) = wrapToPi(x_p(6)); % for each time-step, save estimate and covariance x_est(:,i) = x_p; P(:,:,i) = P_p; S(:,:,i) = 0.5*(S_p + S_p'); end end

function [x, y] = GenerateTruth(x0, u, P0, Q, R, Dt, steps, wrapOn) opt = odeset('RelTol',1e-6,'AbsTol',1e-6); useChol = true; n = size(x0,1); p = size(R,1); x = zeros(n,steps+1); y = zeros(p,steps+1); % set initial conditions dx = mvnrnd(zeros(1,n),P0); x(:,1) = x0 + dx'; x(3,1) = wrapToPi(x(3,1)); x(6,1) = wrapToPi(x(6,1)); for i = 2:steps+1 % generate noisy state if useChol==true wk = chol(Q)*randn(n,1); else wk = mvnrnd(zeros(1,n),Q)'; end [~,next_x] = ode45(@NL_DynModel, [0 Dt], x(:,i-1)', opt, u', wk); if wrapOn == true % wrap angle to [-pi pi] next_x(3) = wrapToPi(next_x(3)); next_x(6) = wrapToPi(next_x(6)); end x(:,i) = next_x(end,:)'; end for i = 1:steps+1 % generate noisy measurement if useChol==true vk = chol(R)*randn(p,1); else vk = mvnrnd(zeros(1,p),R)'; end y(:,i) = NL_MeasModel(x(:,i), vk); if wrapOn == true % wrap angle to [-pi pi] y(1,i) = wrapToPi(y(1,i)); y(3,i) = wrapToPi(y(3,i)); end end end

function [NEES, NIS] = CalcStats(EX, EY, P, S) steps = size(EX, 2); runs = size(EX, 3); NEES_all = zeros(runs,steps); NIS_all = zeros(runs,steps); NEES = zeros(1,steps); NIS = zeros(1,steps); for run=1:runs for step=1:steps NEES(step) = EX(:,step, run)' / P(:,:,step, run) * EX(:,step, run); NIS(step) = EY(:,step, run)' / S(:,:,step, run) * EY(:,step, run); end NEES_all(run,:) = NEES; NIS_all(run,:) = NIS; end % calculate mean at each time step for i=1:steps NEES(i) = mean(NEES_all(:,i)); NIS(i) = mean(NIS_all(:,i)); end end

function PlotMeasurements(hdl,t,y, title) figure(hdl) ftSize = 10; sgtitle(title,'FontSize',ftSize+2,'Interpreter','latex') subplot(3,2,1) plot(t,y(1,:)) ylabel('$\gamma_{ag}$ (rad)','FontSize',ftSize,'Interpreter','latex') xlabel('Time(s)','FontSize',ftSize,'Interpreter','latex') axis([min(t) max(t) min(y(1,:)) max(y(1,:))]) grid on subplot(3,2,2) plot(t,y(2,:)) ylabel('$\rho_{ga}$ (m)','FontSize',ftSize,'Interpreter','latex') xlabel('Time(s)','FontSize',ftSize,'Interpreter','latex') axis([min(t) max(t) min(y(2,:)) max(y(2,:))]) grid on subplot(3,2,3) plot(t,y(3,:)) ylabel('$\gamma_{ga}$ (rad)','FontSize',ftSize,'Interpreter','latex') xlabel('Time(s)','FontSize',ftSize,'Interpreter','latex') axis([min(t) max(t) min(y(3,:)) max(y(3,:))]) grid on subplot(3,2,4) plot(t,y(4,:)) ylabel('$\xi_a$ (m)','FontSize',ftSize,'Interpreter','latex') xlabel('Time(s)','FontSize',ftSize,'Interpreter','latex') axis([min(t) max(t) min(y(4,:)) max(y(4,:))]) grid on subplot(3,2,[5,6]) plot(t,y(5,:)) ylabel('$\eta_a$ (m)','FontSize',ftSize,'Interpreter','latex') xlabel('Time(s)','FontSize',ftSize,'Interpreter','latex') axis([min(t) max(t) min(y(5,:)) max(y(5,:))]) grid on end

function PlotNees(hdl, epsNEESbar, Nsimruns, n, alpha) figure(hdl); Nnx = Nsimruns*n; %%compute intervals: r1x = chi2inv(alpha/2, Nnx )./ Nsimruns; r2x = chi2inv(1-alpha/2, Nnx )./ Nsimruns; figure(hdl) plot(epsNEESbar,'ro','MarkerSize',6,'LineWidth',2),hold on plot(r1x*ones(size(epsNEESbar)),'r--','LineWidth',2) plot(r2x*ones(size(epsNEESbar)),'r--','LineWidth',2) ylabel('NEES statistic, $\bar{\epsilon}_x$','FontSize',14,'Interpreter','latex') xlabel('time step, k','FontSize',14) title('NEES Estimation Results','FontSize',14) legend('NEES @ time k', 'r_1 bound', 'r_2 bound') end

function PlotNis(hdl, epsNISbar, Nsimruns, p, alpha) figure(hdl); Nny = Nsimruns*p; %%compute intervals: r1y = chi2inv(alpha/2, Nny )./ Nsimruns; r2y = chi2inv(1-alpha/2, Nny )./ Nsimruns; plot(epsNISbar,'bo','MarkerSize',6,'LineWidth',2),hold on plot(r1y*ones(size(epsNISbar)),'b--','LineWidth',2) plot(r2y*ones(size(epsNISbar)),'b--','LineWidth',2) ylabel('NIS statistic, $\bar{\epsilon}_y$','FontSize',14,'Interpreter','latex') xlabel('time step, k','FontSize',14) title('NIS Estimation Results','FontSize',14) legend('NIS @ time k', 'r_1 bound', 'r_2 bound') end

function PlotStates(hdl,t,x, title, P) if nargin > 4 p = zeros(size(x)); for ind = 1:size(x,2) p(:,ind) = 2*sqrt(diag(P(:,:,ind))); end displayError = true; else displayError = false; end figure(hdl) ftSize = 10; sgtitle(title,'FontSize',ftSize+2,'Interpreter','latex') subplot(3,2,1) state = 1; plot(t,x(state,:)) if displayError == true hold all, plot(p(state,:),'b--'), plot(-p(state,:),'b--'), hold off end ylabel('$\xi_g$ (m)','FontSize',ftSize,'Interpreter','latex') xlabel('Time(s)','FontSize',ftSize,'Interpreter','latex') axis([min(t) max(t) min(x(state,:)) ... max(x(state,:))]) grid on subplot(3,2,3) state = state + 1; plot(t,x(state,:)) if displayError == true hold all, plot(p(state,:),'b--'), plot(-p(state,:),'b--'), hold off end ylabel('$\eta_g$ (m)','FontSize',ftSize,'Interpreter','latex') xlabel('Time(s)','FontSize',ftSize,'Interpreter','latex') axis([min(t) max(t) min(x(state,:)) ... max(x(state,:))]) grid on subplot(3,2,5) state = state + 1; plot(t,wrapToPi(x(state,:))) if displayError == true hold all, plot(p(state,:),'b--'), plot(-p(state,:),'b--'), hold off end ylabel('$\theta_g$ (rad)','FontSize',ftSize,'Interpreter','latex') xlabel('Time(s)','FontSize',ftSize,'Interpreter','latex') axis([min(t) max(t) min(wrapToPi(x(state,:))) ... max(wrapToPi(x(state,:)))]) grid on subplot(3,2,2) state = state + 1; plot(t,x(state,:)) if displayError == true hold all, plot(p(state,:),'b--'), plot(-p(state,:),'b--'), hold off end ylabel('$\xi_a$ (m)','FontSize',ftSize,'Interpreter','latex') xlabel('Time(s)','FontSize',ftSize,'Interpreter','latex') axis([min(t) max(t) min(x(state,:)) ... max(x(state,:))]) grid on subplot(3,2,4) state = state + 1; plot(t,x(state,:)) if displayError == true hold all, plot(p(state,:),'b--'), plot(-p(state,:),'b--'), hold off end ylabel('$\eta_a$ (m)','FontSize',ftSize,'Interpreter','latex') xlabel('Time(s)','FontSize',ftSize,'Interpreter','latex') axis([min(t) max(t) min(x(state,:)) ... max(x(state,:))]) grid on subplot(3,2,6) state = state + 1; plot(t,wrapToPi(x(state,:))) if displayError == true hold all, plot(p(state,:),'b--'), plot(-p(state,:),'b--'), hold off end ylabel('$\theta_a$ (rad)','FontSize',ftSize,'Interpreter','latex') xlabel('Time(s)','FontSize',ftSize,'Interpreter','latex') axis([min(t) max(t) min(wrapToPi(x(state,:))) ... max(wrapToPi(x(state,:)))]) grid on end