# [ASEN 5044] # Statistical Estimation of Dynamical Systems # Progress Report 2 Fall 2020 Group Members: Joshua Malburg Junior Sundar

# Question #4

## Implementing Linearized Kalman Filter (LKF)

To implement the LKF for this Cooperative Localization problem we define an LKF function which performs the prediction [Equation ( 1 )] and correction [Equation ( 2 )] step.

The function takes in:

- the ground truth values (state dynamics and measurements),

- the nominal state and measurement trajectories without process or measurement noise,

- the process noise covariance (QKF) for tuning and,

- initial values for δx+0 and P+0.

The function outputs:

- the state estimates (xestimate=xnominal+δx+),

- measurement estimates yestimatek+1=ynominalk+1+Hk+1δxk+1-,

- estimation errors (ex=xtruth-xestimate and ey=ttruth-yestimate) and,

- sequence of covariance matrices for state and measurement estimates (P+ and S-)

The LKF runs the following set of equations in a loop for k steps:

|  | δxk+1-=Fkδxk+ Pk+1-=FkPk+FKT+ΩkQkΩkT | ( 1 ) |
| --- | --- | --- |

|  | Sk+1-=Hk+1Pk+1-Hk+1T+Rk+1 Kk+1=Pk+1-Hk+1TSk+1--1 δyk+1=ytruthk+1-ynominalk+1 δxk+1+=δxk+1-+Kk+1δyk+1-Hk+1δxk+1- Pk+1+=In-Kk+1Hk+1Pk+1- | ( 2 ) |
| --- | --- | --- |

This function is included in a Monte Carlo run which feeds the ground truth models, nominal state and measurement trajectories, and the DT state space matrices linearized around the nominal trajectories (Fk,Hk,Ωk) according to the methods from Question #2 and Question #3.

## Truth Model Testing (TMT) for LKF

For the TMT we do 50 Monte Carlo (N=50) runs. This is an appropriate number of Monte Carlo runs as we will have enough data sets for performing an unbiased NEES and NIS test.

The truth model is a simulated run of the nonlinear model with the process and measurement noise generated using the covariances uploaded on Canvas. We seed xtruth0 for the ground truth values for every Monte Carlo run from dx+0~0,P+0 and then xtruth0=xnominal+dx+0 and then for every subsequent values the process noise is obtained from covariance matrix ‘Qtrue’.

Additionally, for the multiple runs in the TMT we feed the LKF the following initial values:

|  | δx+0=xtruth0-xnominal0 P+0=diag110.025110.025 | ( 3 ) |
| --- | --- | --- |

These initial conditions were selected for the following reasons: we are given the nominal and truth state trajectories, and hence the initial perturbation can be derived from those values; furthermore, we have a sufficient degree of certainty of our initial state perturbation such that the state perturbation covariance is finite (P+0≠∞). And also, the angles cannot exceed 180o or go below -180o in radians.

In addition to this, in the LKF, the covariance matrix for the measurement noise (R) is set to be the one uploaded on canvas, i.e. Rtrue. As that information is generally known to us from the sensors being used.

## Tuning

There are two aspects that we need to keep in mind while tuning the Kalman filter. First, we need to make sure that the Kalman filter ‘works properly’, i.e. the estimate error averages at 0, and the 2σ bounds are converging. Second, we also look at the NEES and NIS chi-square tests and make sure they are within the confidence intervals.

We calculate the confidence intervals on MATLAB with significance level α=0.01. We chose this significance level to provide a less stringent condition for proving whether the LKF is doing its job (i.e. having a low enough false-alarm probability). This was because we found that the LKF is not good enough for estimating this nonlinear system. The bounds are calculated using the MATLAB function ‘chi2inv’. As a result, for N=50 Monte Carlo runs, n=6 states, p=5 measured values, the chi-square confidence bounds are:

|  | 4.8133≤ϵx,k≤7.3369 3.9232≤ϵy,k≤6.2269 | ( 4 ) |
| --- | --- | --- |

The ϵx,k and ϵy,k values are obtained by averaging the ϵx,k and ϵy,k values are all k steps, which are obtained from the following equations:

ϵx,k=ex,kTP+-1ex,k

ϵy,k=ey,kTS--1ey,k

While tuning the LKF we vary our predicted process noise covariance matrix (QKF) until the two conditions in the beginning of this subsection are satisfied. We first start with QKF=I6×6 and run the truth model tests and check if the KF works (by looking at the state and measurement estimate errors) and the NEES and NIS tests are satisfied. The result showed that the neither of the conditions were met, hence we moved to QKF=0.1I6×6 then to QKF=0.01I6×6 and then fine-tuned the main diagonal parameters in this way until the conditions were sufficiently satisfied.

In the end, the process noise covariance matrix for the LKF that best satisfied the conditions was found to be:

|  | QKF=diag0.00010.00010.010.10.10.01 | ( 5 ) |
| --- | --- | --- |

Before we move forward, the ‘best satisfying QKF’ is used lightly because, over multiple trials, it was found that the LKF cannot successfully estimate the states for this problem even after multiple variations. It is predicted that more fine tuning is required, however with 6 states to work with and limited time, we think that it is better to look for alternate options.

Referring to Figure 4 and Figure 5, we see that the LKF has done a decent job in estimating the measurements however, every time the ξa value peaks, there is a large estimation error. This error could not be reduced regardless of the corresponding QKF elements selected, and this resulted in the estimation errors carrying over. For instance we γga and ρga also have large estimation errors at the same time points (t≈25s and t≈75s). We know from the linearization strategy in Question #1 and Question #2, that the UGV’s states are observed by feeding the UAV’s sensed values through trigonometric equations. And because of the large estimation errors in ξa, the UGV’s states are adversely affected.

Referring now to Figure 2 and Figure 3. At time ~25s, where the first large deviation in ξa occurs, the estimates for ηg and ξg shift greatly from the ‘correct path’ while still maintaining the ‘correct pattern’. This means, that the shape of the estimate state curve, matches the ground truth curve, however the estimation deviates from the ground truth. After that, the states ηg and ξg start to recover, however at ~75s again there is a large deviation in ξa from the sensor which causes the states ηg and ξg to shift again. Essentially, because of the error in the sensor estimates for ξa and ηa, the state estimates for the UGV are being affected. The cause for the large deviation will be hypothesized and discussed in the next subsection.

Although, the angle estimates θg and θa are not significantly affected as they are more dependent on the control inputs. However we do see intermitted deviations at the 25s and 75s marks in the state estimates for θa in Figure 3.

Now while the state estimation errors for ηg and ξg do not converge, their covariances do. In fact, all the covariances of the state estimates (P+) and measurement estimates (S-) converge.

We will not look at the NEES and NIS tests for the LKF. Referring to Figure 1, we can see that the NIS test is, too and extent, being satisfied, however at the 25s and 75s marks the points go beyond the r1 and r2 limits. This overlaps with the previous observation of how the measurement estimates for ξa deviate significantly at those times.

We can also see how the effect of this deviation carries over to the state estimate errors by looking at the NEES test where the points are within the confidence interval in the beginning but very quickly they go beyond the limit.

This tells us that the current configuration for the LKF fails the NEES and NIS tests. To reiterate, there is still room for more fine tuning of the LKF, but we believe that rather than wasting more time in tuning this further, a better alterative would be preferred.

## Comment on LKF for Estimating Nonlinear System

The tuning procedure was arduous and complicated. However, after multiple attempts, we can conclude that the LKF is not a good enough filter to estimate the nonlinear system for cooperative localization.

We derived this conclusion because we were not able to tune the LKF to satisfy the two conditions: the LKF ‘works properly’ and satisfies the NEES and NIS chi-square tests.

Referring to the UAV’s states, it seems that the LKF starts to lose accuracy in its states when the aircraft is moving near the boundaries of its domain in and ξa. After some analysis, we believe that this is because of the large covariance for the sensed values ρga, ξa and ηa. The latter two directly correspond to the UAV’s state while ρga contains the UAV’s states and UGV’s states combined, the resulting ground truth measurement values (with the synthesized measurement noise) are believed to deviate beyond the bounds of the nominal trajectory, especially when the UAV is near the edge of its domain.

As a result, the LKF is not able to correctly estimate the measurement perturbations δy- at the time 25s and 75s when the UAV is at the farthest point, which carries over to the corrected state perturbation estimate δx+.

We believe, a better sensor could improve the state estimates obtained from the LKF. However, the issue of deviating beyond the nominal trajectory can be avoided by using the EKF which calculates the nominal trajectory regularly from the nonlinear model.

We also tested the filter by checking whether angle wrapping between operations (for γga, γag, θg and θa) effected or improved the results (i.e. wrapping the nominal and ground truths before feeding into LKF). The result was worse off, because the filter would fail to estimate the states at the point where the angle wrapped back to within the -π,π intervals. Hence angles are not wrapped until after the estimate is obtained through the filter.

The MATLAB codes for the LKF implementation, the Monte Carlo runs, and the NEES and NIS tests are attached in Appendix D.

Figure 1 – NEES and NIS chi-square test for N=50 LKF Monte Carlo runs with Equations ( 3 )( 4 )( 5 ) applied

Figure 2 – State estimate errors and 2σ bounds of sample LKF Monte Carlo run with Equations ( 3 )( 4 )( 5 ) applied

Figure 3 – State estimates and state ground truth of sample LKF Monte Carlo run with Equations ( 3 )( 4 )( 5 ) applied

Figure 4 – Measurement estimates and ground truth of sample LKF Monte Carlo run with Equations ( 3 )( 4 )( 5 ) applied

Figure 5 – Measurement estimates and ground truth of sample LKF Monte Carlo run with Equations ( 3 )( 4 )( 5 ) applied

# Question #5

## Extended Kalman Filter (EKF)

The EKF starts off at time-step zero (0) with an estimate of the total state and covariance.  At each time-step k, during the prediction step the EKF uses the non-linear equations to calculate the estimated state which in turn is used to approximate the predicted covariance matrix.

xk+1-=fxk+,uk,wk=0

Pk+1-=FkPk+FkT+ΩkQkΩkT

where			Fkxk+,uk,wk=0≈I+∆T∙A(xk+,uk(tk),wk(tk)=0)

Ωk≈∆T∙Γ(t)(t=tk)

During the correction step the EKF calculates the innovation vector (yk+1-yk+1-) using the measurement estimate derived from the non-linear measurement equation.

yk+1-=hxk+1-,vk+1=0

eyk+1=yk+1-yk+1-

The filter then generates a new linearized DT System (H) matrix, linearized about the new state estimate, to calculate the Kalman gain, update the total state estimate and update the covariance matrix.

Hk+1=δhδxxk+1-

Kk+1=Pk+1-Hk+1THk+1Pk+1-Hk+1T+Rk+1-1

xk+1+=xk+1-+Kk+1eyk+1

MATLAB code for implementation and tuning of the EKF can be found in Appendix E.

## Tuning Approach

Tuning the filter involved generating randomized ground truth data from many Monte Carlo simulations and calculating the mean NEES and NIS values at each time step.  The EKF’s process noise matrix Q was then tweaked until the NEES and NIS consistency tests were within the desired accuracy bounds (alpha = ±5%).  The initial state was also randomized to demonstrate the filter wasn’t biased to a specific nominal trajectory.

## Typical Simulation Plots

The section shows plots from a typical simulation run. Figure 6captures an example of the randomly generated UGV and UAV state data.

Figure 6 - Simulation Run - Ground Truth States

Figure 7 shows an example of the randomly generated noisy measurement data.

Figure 7 - Simulation Run - Simulated Measurement Data

Figure 8 – Estimated State Errors

Figure 9 - Estimated Measurement Errors

## Chi-Square Test Results

This section shows the final NEES and NIS test results.

Figure 10 - Final NEES results

Figure 11 - Final NIS Results

Appendix D – Codes accompanying Question 4

function [x_est,y_est,dx,P,S,e_x,e_y] = LKF(dx_init,P_init,x_nom,y_nom,x,y,Fk,Hk,Ok,Q,R) n = size(Fk(:,:,1),2); % No. of States p = size(Hk(:,:,1),1); % No. of Measurements steps = length(x_nom(1,:))-1; % No. of k steps dx = zeros(n,steps+1); P = zeros(n,n,steps+1); S = zeros(p,p,steps+1); K = zeros(n,p,steps+1); dx(:,1) = dx_init; % initial dx dy = y-y_nom; % ground truth dx P(:,:,1) = P_init; % inital P I = eye(n); y_est = zeros(5,steps+1); y_est(:,1) = y_nom(:,1) + Hk(:,:,1)*(dx(:,1)); e_y = zeros(p,steps+1); e_y(:,1) = y(:,1) - y_est(:,1);% error in measurement estimates S(:,:,1) = Hk(:,:,1)*P(:,:,1)*Hk(:,:,1)' + R; K(:,:,1) = P(:,:,1)*Hk(:,:,1)'*inv(S(:,:,1)); for i=1:steps % Prediction Step dx(:,i+1) = Fk(:,:,i)*dx(:,i); P(:,:,i+1) = Fk(:,:,i)*P(:,:,i)*Fk(:,:,i)' + Ok(:,:,i)*Q*Ok(:,:,i)'; % Correction Step S(:,:,i+1) = Hk(:,:,i+1)*P(:,:,i+1)*Hk(:,:,i+1)' + R; K(:,:,i+1) = P(:,:,i+1)*Hk(:,:,i+1)'*inv(S(:,:,i+1)); y_est(:,i+1) = y_nom(:,i) + Hk(:,:,i+1)*(dx(:,i+1)); e_y(:,i+1) = y(:,i+1) - y_est(:,i+1); dx(:,i+1) = dx(:,i+1) + K(:,:,i+1)*(dy(:,i+1) - Hk(:,:,i+1)*dx(:,i+1)); P(:,:,i+1) = (I - K(:,:,i+1)*Hk(:,:,i+1))*P(:,:,i+1); end x_est = x_nom + dx; e_x = x - x_est; end

function [x_truth,y_truth,x_est,y_est,dx,P,S,e_x,e_y] = LKF_MonteCarlo(Q,R,steps) load("cooplocalization_finalproj_KFdata.mat"); x_nom = [10 0 pi/2 -60 0 -pi/2]'; u_nom = [2 -pi/18 12 pi/25]'; x_pert = [0 1 0 0 0 0.1]'; Dt = 0.1; n = size(x_nom,1); P_init = diag([1 1 0.025 1 1 0.025]); t = 0:Dt:steps*Dt; % Generate truth model outputs for nominal trajectories [x_truth, y_truth] = GenerateTruth(x_nom, u_nom, P_init, Qtrue, Rtrue, Dt, steps, false); % Generate nominal trajectories [~,x_NL] = ode45(@(t,x) NL_DynModel(t,x,u_nom,zeros(6,1)),t,x_nom); x_NL = x_NL'; y_NL = zeros(5,length(t)); for i=1:length(t) y_NL(:,i) = NL_MeasModel(x_NL(:,i),zeros(5,1)); end % Generate DT matrices along nominal trajectory x_nominal = x_NL; y_nominal = y_NL; Fk = zeros(6,6,length(t)); Hk = zeros(5,6,length(t)); Ok = zeros(6,6,length(t)); for i=1:length(t) [A_t, B_t, C_t] = Linearize(x_nominal(:,i),u_nom); [Fk(:,:,i), ~ , Hk(:,:,i)] = Discretize(A_t,B_t,C_t, Dt); Ok(:,:,i) = eye(6); end % Run LKF on Data dx_init = x_truth(:,1)-x_nominal(:,1); P_init = eye(6); [x_est,y_est,dx,P,S,e_x,e_y] = LKF(dx_init,P_init,x_nominal,y_nominal,x_truth,y_truth,Fk,Hk,Ok,Q,R); end

clc clear load('cooplocalization_finalproj_KFdata.mat') seed = 100; rng(seed); Dt = 0.1; steps = 1000; n = 6; p = 5; t = 0:Dt:steps*Dt; N = 50; % No. of Monte Carlo runs NEES_all = zeros(N,steps+1); NIS_all = zeros(N,steps+1); % Tuning parameters Q = diag([0.0001 0.0001 0.01 0.1 0.1 0.01]); R = Rtrue; for i=1:N disp(i) NEES = zeros(1,steps+1); NIS = zeros(1,steps+1); [x_truth,y_truth,x_est,y_est,dx,P,S,e_x,e_y] = LKF_MonteCarlo(Q,R,steps); for k=1:steps+1 NEES(:,k) = e_x(:,k)'*inv(P(:,:,k))*e_x(:,k); NIS(:,k) = e_y(:,k)'*inv(S(:,:,k))*e_y(:,k); end NEES_all(i,:) = NEES; NIS_all(i,:) = NIS; end NEES_bar = zeros(1,steps+1); NIS_bar = zeros(1,steps+1); for i=1:steps+1 NEES_bar(1,i) = mean(NEES_all(:,i)); NIS_bar(1,i) = mean(NIS_all(:,i)); end alpha = 0.01; rx1 = (chi2inv(alpha/2,N*n))./N; rx2 = (chi2inv(1-alpha/2,N*n))./N; ry1 = (chi2inv(alpha/2,N*p))./N; ry2 = (chi2inv(1-alpha/2,N*p))./N;

Appendix E – Codes accompanying Question 5

% Question 5 - Tune EKF close all, clearvars, clc load("cooplocalization_finalproj_KFdata.mat"); x0 = [10 0 pi/2 -60 0 -pi/2]'; u0 = [2 -pi/18 12 pi/25]'; Dt = 0.1; n = size(x0,1); steps = 1000; seed = 100; rng(seed); Q = diag([.0015, .0015, 0.01, 0.001, 0.005, 0.01]); P0 = diag([1 1 0.025 1 1 0.025]); runs = 100; EX = zeros(n, steps+1, runs); p = 5; EY = zeros(p, steps+1, runs); PS = zeros(n, n, steps+1, runs); SS = zeros(p, p, steps+1, runs); fig1 = figure(1); fig2 = figure(2); fig3 = figure(3); fig4 = figure(4); enablePlotDuring = true; for run = 1:runs disp(['run #', num2str(run)]); % generate truth for run [x, y] = GenerateTruth(x0, u0, P0, Qtrue, Rtrue, Dt, steps, true); t = (0:(length(x)-1))*Dt; % assume we can get exact measurement noise from % specifications of sensors R = Rtrue; % Run filter for all time-steps of run #k [x_est, y_est, P, S] = EKF(x0, P0, u0, y, Q, R, Dt); % wrap angle diff too!! ex = x - x_est; ex(3,:) = angdiff(x_est(3,:),x(3,:)); ex(6,:) = angdiff(x_est(6,:),x(6,:)); ey = y - y_est; ey(1,:) = angdiff(y_est(1,:),y(1,:)); ey(3,:) = angdiff(y_est(3,:),y(3,:)); % Plot error during monte carlo runs if enablePlotDuring == true PlotStates(fig1,t,ex, ['State Errors, Run ',num2str(run)], P); PlotMeasurements(fig2,t,y,'Ground Truth Measurements'); PlotStates(fig3,t,x,'Ground Truth States'); PlotMeasurements(fig4,t,ey,['Ground Truth Measurement Errors, Run ',num2str(run)]); end % save run data from NEES/NIS tests EX(:, :, run) = ex; EY(:, :, run) = ey; PS(:, :, :, run) = P; SS(:, :, :, run) = S; end %% Calculate NEES and NIS statistics [NEES_bar, NIS_bar] = CalcStats(EX, EY, PS, SS); %-------------------------------------------------------------------------- % Plots for (a) PlotStates(fig1,t,x - x_est, ['State Errors, Run ',num2str(run)], P); PlotMeasurements(fig2,t,y,'Ground Truth Measurements'); PlotStates(fig3,t,x,'Ground Truth States'); PlotMeasurements(fig4,t,y - y_est,['Ground Truth Measurement Errors, Run ',num2str(run)]); %-------------------------------------------------------------------------- % Plots for (b) fig5 = figure(5); alpha = 0.05; PlotNees(fig5, NEES_bar, runs, n, alpha); %-------------------------------------------------------------------------- % Plots for (c) fig6 = figure(6); PlotNis(fig6, NIS_bar, runs, p, alpha);

function [x_est, y_est, P, S] = EKF(x0, P0, u, y, Q, R, Dt) % set simulation tolerances for ode45 opt = odeset('RelTol',1e-6,'AbsTol',1e-6); n = size(x0, 1); % number of states p = size(R, 1); % number of measurements steps = size(y,2); % number of time steps x_est = zeros(n, steps); % state estimate vector y_est = zeros(p, steps); % measurement estimate vector P = zeros(n, n, steps); % coveriance S = zeros(p, p, steps); % start with initial estimate of total state % and covariance x_p = x0; P_p = P0; for i=1:steps %---------------------------------------------------------- % Prediction Step % use full NL equations to estimate state at next time step % using state at previous time step; since Wk is AWGN, % its expected value is zero, set input to zero wk = zeros(1,n); [~, x_m] = ode45(@NL_DynModel, [0.0 Dt], x_p', opt, u', wk); x_m = x_m(end,:)'; x_m(3) = wrapToPi(x_m(3)); x_m(6) = wrapToPi(x_m(6)); % to calculate covariance, linearize "online" % about current state estimate [A_t,B_t,C_t] = Linearize(x_m, u); [F, ~, H] = Discretize(A_t, B_t ,C_t, Dt); P_m = F*P_p*F' + Q; x_m(3) = wrapToPi(x_m(3)); x_m(6) = wrapToPi(x_m(6)); % uese estimated state from NL ODEs; since Wk is AWGN, % its expected value is zero, set to zero vk = zeros(p,1); y_est(:,i) = NL_MeasModel(x_m, vk); y_est(1,i) = wrapToPi(y_est(1,i)); y_est(3,i) = wrapToPi(y_est(3,i)); % calculate innovation vector e_y = y(:,i) - y_est(:,i); e_y(1) = wrapToPi(e_y(1)); e_y(3) = wrapToPi(e_y(3)); %---------------------------------------------------------- % Correction Step % calculate gain using linearized measurement % matrix H and covariance from Prediction Step S_p = H*P_m*H' + R; K = P_m*H'/S_p; % calculate posterior state estimate and covariance x_p = x_m + K*e_y; P_p = (eye(n) - K*H)*P_m; x_p(3) = wrapToPi(x_p(3)); x_p(6) = wrapToPi(x_p(6)); % for each time-step, save estimate and covariance x_est(:,i) = x_p; P(:,:,i) = P_p; S(:,:,i) = 0.5*(S_p + S_p'); end end

function [x, y] = GenerateTruth(x0, u, P0, Q, R, Dt, steps, wrapOn) opt = odeset('RelTol',1e-6,'AbsTol',1e-6); useChol = true; n = size(x0,1); p = size(R,1); x = zeros(n,steps+1); y = zeros(p,steps+1); % set initial conditions dx = mvnrnd(zeros(1,n),P0); x(:,1) = x0 + dx'; x(3,1) = wrapToPi(x(3,1)); x(6,1) = wrapToPi(x(6,1)); for i = 2:steps+1 % generate noisy state if useChol==true wk = chol(Q)*randn(n,1); else wk = mvnrnd(zeros(1,n),Q)'; end [~,next_x] = ode45(@NL_DynModel, [0 Dt], x(:,i-1)', opt, u', wk); if wrapOn == true % wrap angle to [-pi pi] next_x(3) = wrapToPi(next_x(3)); next_x(6) = wrapToPi(next_x(6)); end x(:,i) = next_x(end,:)'; end for i = 1:steps+1 % generate noisy measurement if useChol==true vk = chol(R)*randn(p,1); else vk = mvnrnd(zeros(1,p),R)'; end y(:,i) = NL_MeasModel(x(:,i), vk); if wrapOn == true % wrap angle to [-pi pi] y(1,i) = wrapToPi(y(1,i)); y(3,i) = wrapToPi(y(3,i)); end end end

function [NEES, NIS] = CalcStats(EX, EY, P, S) steps = size(EX, 2); runs = size(EX, 3); NEES_all = zeros(runs,steps); NIS_all = zeros(runs,steps); NEES = zeros(1,steps); NIS = zeros(1,steps); for run=1:runs for step=1:steps NEES(step) = EX(:,step, run)' / P(:,:,step, run) * EX(:,step, run); NIS(step) = EY(:,step, run)' / S(:,:,step, run) * EY(:,step, run); end NEES_all(run,:) = NEES; NIS_all(run,:) = NIS; end % calculate mean at each time step for i=1:steps NEES(i) = mean(NEES_all(:,i)); NIS(i) = mean(NIS_all(:,i)); end end

function PlotMeasurements(hdl,t,y, title) figure(hdl) ftSize = 10; sgtitle(title,'FontSize',ftSize+2,'Interpreter','latex') subplot(3,2,1) plot(t,y(1,:)) ylabel('$\gamma_{ag}$ (rad)','FontSize',ftSize,'Interpreter','latex') xlabel('Time(s)','FontSize',ftSize,'Interpreter','latex') axis([min(t) max(t) min(y(1,:)) max(y(1,:))]) grid on subplot(3,2,2) plot(t,y(2,:)) ylabel('$\rho_{ga}$ (m)','FontSize',ftSize,'Interpreter','latex') xlabel('Time(s)','FontSize',ftSize,'Interpreter','latex') axis([min(t) max(t) min(y(2,:)) max(y(2,:))]) grid on subplot(3,2,3) plot(t,y(3,:)) ylabel('$\gamma_{ga}$ (rad)','FontSize',ftSize,'Interpreter','latex') xlabel('Time(s)','FontSize',ftSize,'Interpreter','latex') axis([min(t) max(t) min(y(3,:)) max(y(3,:))]) grid on subplot(3,2,4) plot(t,y(4,:)) ylabel('$\xi_a$ (m)','FontSize',ftSize,'Interpreter','latex') xlabel('Time(s)','FontSize',ftSize,'Interpreter','latex') axis([min(t) max(t) min(y(4,:)) max(y(4,:))]) grid on subplot(3,2,[5,6]) plot(t,y(5,:)) ylabel('$\eta_a$ (m)','FontSize',ftSize,'Interpreter','latex') xlabel('Time(s)','FontSize',ftSize,'Interpreter','latex') axis([min(t) max(t) min(y(5,:)) max(y(5,:))]) grid on end

function PlotNees(hdl, epsNEESbar, Nsimruns, n, alpha) figure(hdl); Nnx = Nsimruns*n; %%compute intervals: r1x = chi2inv(alpha/2, Nnx )./ Nsimruns; r2x = chi2inv(1-alpha/2, Nnx )./ Nsimruns; figure(hdl) plot(epsNEESbar,'ro','MarkerSize',6,'LineWidth',2),hold on plot(r1x*ones(size(epsNEESbar)),'r--','LineWidth',2) plot(r2x*ones(size(epsNEESbar)),'r--','LineWidth',2) ylabel('NEES statistic, $\bar{\epsilon}_x$','FontSize',14,'Interpreter','latex') xlabel('time step, k','FontSize',14) title('NEES Estimation Results','FontSize',14) legend('NEES @ time k', 'r_1 bound', 'r_2 bound') end

function PlotNis(hdl, epsNISbar, Nsimruns, p, alpha) figure(hdl); Nny = Nsimruns*p; %%compute intervals: r1y = chi2inv(alpha/2, Nny )./ Nsimruns; r2y = chi2inv(1-alpha/2, Nny )./ Nsimruns; plot(epsNISbar,'bo','MarkerSize',6,'LineWidth',2),hold on plot(r1y*ones(size(epsNISbar)),'b--','LineWidth',2) plot(r2y*ones(size(epsNISbar)),'b--','LineWidth',2) ylabel('NIS statistic, $\bar{\epsilon}_y$','FontSize',14,'Interpreter','latex') xlabel('time step, k','FontSize',14) title('NIS Estimation Results','FontSize',14) legend('NIS @ time k', 'r_1 bound', 'r_2 bound') end

function PlotStates(hdl,t,x, title, P) if nargin > 4 p = zeros(size(x)); for ind = 1:size(x,2) p(:,ind) = 2*sqrt(diag(P(:,:,ind))); end displayError = true; else displayError = false; end figure(hdl) ftSize = 10; sgtitle(title,'FontSize',ftSize+2,'Interpreter','latex') subplot(3,2,1) state = 1; plot(t,x(state,:)) if displayError == true hold all, plot(p(state,:),'b--'), plot(-p(state,:),'b--'), hold off end ylabel('$\xi_g$ (m)','FontSize',ftSize,'Interpreter','latex') xlabel('Time(s)','FontSize',ftSize,'Interpreter','latex') axis([min(t) max(t) min(x(state,:)) ... max(x(state,:))]) grid on subplot(3,2,3) state = state + 1; plot(t,x(state,:)) if displayError == true hold all, plot(p(state,:),'b--'), plot(-p(state,:),'b--'), hold off end ylabel('$\eta_g$ (m)','FontSize',ftSize,'Interpreter','latex') xlabel('Time(s)','FontSize',ftSize,'Interpreter','latex') axis([min(t) max(t) min(x(state,:)) ... max(x(state,:))]) grid on subplot(3,2,5) state = state + 1; plot(t,wrapToPi(x(state,:))) if displayError == true hold all, plot(p(state,:),'b--'), plot(-p(state,:),'b--'), hold off end ylabel('$\theta_g$ (rad)','FontSize',ftSize,'Interpreter','latex') xlabel('Time(s)','FontSize',ftSize,'Interpreter','latex') axis([min(t) max(t) min(wrapToPi(x(state,:))) ... max(wrapToPi(x(state,:)))]) grid on subplot(3,2,2) state = state + 1; plot(t,x(state,:)) if displayError == true hold all, plot(p(state,:),'b--'), plot(-p(state,:),'b--'), hold off end ylabel('$\xi_a$ (m)','FontSize',ftSize,'Interpreter','latex') xlabel('Time(s)','FontSize',ftSize,'Interpreter','latex') axis([min(t) max(t) min(x(state,:)) ... max(x(state,:))]) grid on subplot(3,2,4) state = state + 1; plot(t,x(state,:)) if displayError == true hold all, plot(p(state,:),'b--'), plot(-p(state,:),'b--'), hold off end ylabel('$\eta_a$ (m)','FontSize',ftSize,'Interpreter','latex') xlabel('Time(s)','FontSize',ftSize,'Interpreter','latex') axis([min(t) max(t) min(x(state,:)) ... max(x(state,:))]) grid on subplot(3,2,6) state = state + 1; plot(t,wrapToPi(x(state,:))) if displayError == true hold all, plot(p(state,:),'b--'), plot(-p(state,:),'b--'), hold off end ylabel('$\theta_a$ (rad)','FontSize',ftSize,'Interpreter','latex') xlabel('Time(s)','FontSize',ftSize,'Interpreter','latex') axis([min(t) max(t) min(wrapToPi(x(state,:))) ... max(wrapToPi(x(state,:)))]) grid on end